Network Segmentation Design Document

Authority: Adopted by Tri-River Regional 911 Center under the authority of Dr. Lisa Park, Director. Version: 1 Effective Date: 2026-01-26 Next Review: 2027-01-26 Owner: Information Security Analyst Classification: Restricted

## 1. Purpose

This document sets the agency's network segmentation model: the zones and VLANs, what each isolates, the DMZ and inter-segment rules, and the reasons behind them. It is the design the Firewall Ruleset Documentation implements as-built, and it keeps critical services such as GIS and call routing separated from general-use systems.

## 2. Diagram

## 3. Segmentation Zones

| Zone / VLAN | Purpose / systems in zone | Trust level | Isolated from | Inter-segment rule |
| --- | --- | --- | --- | --- |
| Dispatch VLAN | Call-handling and dispatch (redundant) | Critical | Admin, guest, DMZ | Deny by default; micro-segmented |
| Integration VLANs | GIS, RMS, ALI, video | Critical | General-use, guest | Host-based FW; allow from dispatch only |
| Voice VLAN | SBC / VoIP | Critical | Guest, admin | Allow to SBC only |
| Admin VLAN | Workstations, email | Standard | Critical VLANs | Deny to critical VLANs |
| DMZ | Partner and public services | Untrusted | All internal VLANs | Ingress/egress filtered |
| Guest Wi-Fi | Visitor internet access | Untrusted | All internal | Internet-only; isolated |

## 4. Inter-Segment Rules & Traffic Policy

All inter-zone traffic is denied by default across both centers; only defined flows pass, each through a next-generation firewall or the DMZ proxy. Critical services are micro-segmented with host-based firewalls, and the admin VLAN cannot reach the critical VLANs. Enforced as-built in the Firewall Ruleset Documentation and carried into the System Configuration Baselines.

## 5. Critical Service Isolation

Call routing (the CPE and CAD path) and the NG911 integrations (GIS, RMS, ALI, video) are micro-segmented on their own critical VLANs across both centers, reachable only over the defined flows. A compromise on the administrative or guest network cannot reach these services. Isolation is verified on change and during periodic review.

## Profile Scaling Notes

| PSAP Profile | Minimum-viable implementation |
| --- | --- |
| Large (25+, dedicated IT/security) | A full zoned design with micro-segmentation for critical services, DMZ and internal separation across both centers, and inter-segment rules managed and monitored as part of the security program. |

Source Authority NENA-STA-040.2-2024: §6.9, §6.12, §6.13, §6.16 NIST SP 800-53 Rev. 5: AC-4, CA-9, SC-7

## Approval Signatures

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Director | Dr. Lisa Park | /s/ Dr. Lisa Park | 2026-01-26 |
| Information Security Analyst | Raymond Osei | /s/ Raymond Osei | 2026-01-26 |

## Change History

| Version | Date | Author | Summary of Change |
| --- | --- | --- | --- |
| 1 | 2026-01-26 | Raymond Osei | Initial adoption |