Network Architecture Diagram

Authority: Adopted by Hickory County 911 under the authority of Tracy Holloway, Director. Version: 1 Effective Date: 2026-02-16 Next Review: 2027-02-16 Owner: County IT Contact Classification: Restricted

## 1. Purpose

This diagram is the current, dated map of the agency's network: every segment and VLAN, the devices on each, the boundary points, and every external connection. It is the reference the firewall ruleset, segmentation design, external connection inventory, and monitoring records all build on, so it is kept accurate against the Master Asset Inventory.

## 2. Diagram

## 3. Segments & Devices

| Node / device | Segment / VLAN | Role / function | Trust boundary |
| --- | --- | --- | --- |
| Firewall (FW-01) | Perimeter | Boundary control, default-deny | Perimeter |
| Session border controller (SBC-01) | Voice / edge | SIP boundary to the ESInet | Perimeter |
| CAD / dispatch server | Dispatch VLAN | Call-handling and dispatch | Internal, critical |
| Dispatch workstations (x5) | Dispatch VLAN | Call-taking and dispatch | Internal, critical |
| Admin workstations | Admin VLAN | Administration and email | Internal |
| Guest Wi-Fi access point | Guest | Visitor internet only | Untrusted, isolated |

## 4. Relationship Notes

Traffic between segments is denied by default. The dispatch and admin VLANs reach the internet only through the firewall on 443, and the guest Wi-Fi is internet-only and fully isolated. The ESInet enters through SBC-01 to the firewall. This diagram was last verified against the Master Asset Inventory on 2026-02-16, separate from the document review date. The Firewall Ruleset Documentation carries the as-built rules and the Network Segmentation Design Document sets the intended zoning.

## Profile Scaling Notes

| PSAP Profile | Minimum-viable implementation |
| --- | --- |
| Small (1–5 dispatchers, no in-house IT) | One page is enough: a simple diagram showing the internet or ESInet link, the firewall, the dispatch and admin segments, and the CAD and phone systems. Update it whenever the network changes and re-verify once a year. |

Source Authority NENA-STA-040.2-2024: §6.7.3, §6.9, §6.11, §6.13 NIST SP 800-53 Rev. 5: AC-4, CA-3, CA-9, PL-8, SC-7

## Approval Signatures

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Director | Tracy Holloway | /s/ Tracy Holloway | 2026-02-16 |
| County IT Contact | Marcus Reed | /s/ Marcus Reed | 2026-02-16 |

## Change History

| Version | Date | Author | Summary of Change |
| --- | --- | --- | --- |
| 1 | 2026-02-16 | Marcus Reed | Initial adoption |