Patch Testing Procedure

Authority: Adopted by Hickory County 911 under the authority of Tracy Holloway, Director. Version: 1 Effective Date: 2026-02-16 Next Review: 2027-02-16 Owner: County IT Contact Classification: Internal

## 1. Purpose

This procedure sets out how operating-system, firmware, and application updates are tested in a non-production environment before they reach live dispatch systems. It confirms each update is authentic before installation and intact after deployment, so a bad or tampered update does not disrupt 9-1-1 service.

## 2. Scope

- Applies to: the County IT contact and the Director

- Covers: operating-system, firmware, and application updates for dispatch and production systems, including session border controllers and other specialized gear

- Excludes: emergency changes, which follow the Emergency Change Procedures

## 3. Prerequisites

- A non-production or test environment kept separate from live dispatch systems

- The current patch sources and the vendor signature or hash references used to check authenticity

- Change authorization under the Change Management Policy

## 4. Procedure Steps

| Step | Action | Responsible role |
| --- | --- | --- |
| 1 | Obtain the update from a trusted, authenticated source | County IT |
| 2 | Verify authenticity: check the publisher signature or compare the vendor-published hash before installing | County IT |
| 3 | Apply the update on a spare, non-production machine (no separate lab) | County IT |
| 4 | Test core function and stability: call handling, CAD, and mapping as applicable | County IT / dispatch |
| 5 | Record the test result and obtain change approval | County IT / Director |
| 6 | Deploy to production during an approved maintenance window | County IT |
| 7 | Verify post-deployment integrity with a hash check and review logs for errors | County IT |
| 8 | Record the outcome in the Patch Management Schedule & Tracking Log | County IT |

## 5. Verification Criteria

Every update is authenticity-checked before installation and integrity-verified by hash after deployment; the test result and post-deployment log review are recorded before the change is closed.

## 6. Related Documents

Change Management Policy; Patch Management Schedule & Tracking Log; Configuration Drift Detection Records; Log Integrity Verification Procedures.

## Profile Scaling Notes

| PSAP Profile | Minimum-viable implementation |
| --- | --- |
| Small (1–5 dispatchers, no in-house IT) | No separate lab is expected. Test a critical update on one non-production machine or a spare, confirm the vendor signature first, and keep a fast rollback such as an image or restore point. The county or vendor IT contact runs it and records the result. |

Source Authority NENA-STA-040.2-2024: §5.2, §5.5 NIST SP 800-53 Rev. 5: CM-3, CM-4, CM-14, SI-2, SI-7

## Approval Signatures

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Director | Tracy Holloway | /s/ Tracy Holloway | 2026-02-16 |
| County IT Contact | Marcus Reed | /s/ Marcus Reed | 2026-02-16 |

## Revision History

| Version | Date | Author | Summary of Change |
| --- | --- | --- | --- |
| 1 | 2026-02-16 | Marcus Reed | Initial adoption |