NTP Configuration Documentation

Authority: Adopted by Hickory County 911 under the authority of the Director. Version: 1.0 Effective Date: 2026-04-10 Next Review: 2027-04-10 Owner: County IT Administrator Classification: Restricted

## 1. Purpose and Scope

This record documents the Agency’s time synchronization as built: the authoritative time sources, the systems that synchronize to them, the authentication and acceptable drift, and failover. It supports the Logging Policy’s timestamp requirements and is the baseline the Time Synchronization Audit Records verify against. Scope includes servers, network devices, Session Border Controllers, and edge devices.

## 2. Time Sources and Synchronized Systems

Authoritative time sources

| Source | Type | Stratum / reference | Authentication | Role |
| --- | --- | --- | --- | --- |
| Firewall NTP service | Internal NTP (GPS-fed) | Stratum 2 | Symmetric key | Primary |
| GPS clock | GPS | GPS reference | n/a | Failover |

Synchronized systems

| System or group | Syncs to | Poll interval | Acceptable drift |
| --- | --- | --- | --- |
| CAD server and file server | Firewall NTP | Hourly | Under 1 second |
| Network switches and SBC | Firewall NTP | Hourly | Under 1 second |
| Workstations | Firewall NTP | At logon | Under 2 seconds |

## 3. Current Configuration

| Setting | Current value | Notes |
| --- | --- | --- |
| NTP service | Enabled on all synchronized hosts | Served from the firewall NTP |
| Failover behavior | Automatic to the GPS clock | Holdover a few hours |

## 4. Approved Settings and Deviation Log

| Setting | Approved value | Deviation? | Approved by (role) | Date |
| --- | --- | --- | --- | --- |
| Acceptable drift | Under 1 second for logging hosts | N | County IT Administrator | 2026-04-10 |
| Authentication | Symmetric key on the internal NTP | N | County IT Administrator | 2026-04-10 |

## 5. Validation and Review

The as-built time synchronization is verified against this record at least quarterly by the County IT Administrator; results are recorded in the Time Synchronization Audit Records.

## Profile Scaling Notes

| PSAP Profile | Minimum-viable implementation |
| --- | --- |
| Small (1–5 dispatchers, no in-house IT) | One or two time sources (the firewall or a GPS clock serving NTP), with the handful of servers and the SBC pointed at them. Record the source and the acceptable drift; check it during the periodic time audit. |

Source Authority NENA-STA-040.2-2024: §5.6, §5.6.1 NIST SP 800-53 Rev. 5: AU-8, SC-45, SC-45(1)

## Approval Signatures

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| County IT Administrator | Marcus Reed | /s/ Marcus Reed | 2026-04-10 |
| Director | Tracy Holloway | /s/ Tracy Holloway | 2026-04-10 |

## Change History

| Version | Date | Author | Summary of Change |
| --- | --- | --- | --- |
| 1.0 | 2026-04-10 | County IT Administrator | Initial issue |