System Redundancy Design Documentation

Authority: Adopted by Wabash Valley Communications under the authority of Anthony Reyes, Director. Version: 1.0 Effective Date: 2026-01-15 Next Review: 2027-01-15 Owner: Shared County IT Classification: Restricted

## 1. Purpose

This document maps Wabash Valley Communications' redundancy: dual CAD and dual ESInet with automatic failover across two sites, a backup radio console, and UPS-plus-generator power, so a single failure or vendor outage does not stop call handling.

## 2. Diagram

## 3. Redundant Components

| Service / component | Primary and diverse counterpart | Failover mechanism (auto / manual) | Expected shift time |
| --- | --- | --- | --- |
| Internet / ESInet path | Carrier A (primary); Carrier B (diverse) | Automatic | ~20 sec |
| Call routing / CPE | Primary CPE; redundant CPE node | Automatic | ~1 min |
| Computer-aided dispatch (CAD) | Primary CAD; secondary CAD (hot standby) | Automatic | ~45 sec |
| Radio / dispatch console | Primary console; backup console | Manual | immediate |

## 4. Diversity & Failover Notes

Wabash Valley Communications runs dual CAD and dual ESInet with automatic failover, plus a backup console and UPS-plus-generator power. Primary and secondary systems sit in separate equipment rooms across two sites. The remaining single point of failure, the shared radio backhaul, is carried into the Single-Point-of-Failure Assessment Record. Each failover path is tested in the Failover Test Records; certificate renewal is tracked with the Certificate Inventory so a failover keeps valid certificates.

## Profile Scaling Notes

| PSAP Profile | Minimum-viable implementation |
| --- | --- |
| Small (1–5 dispatchers, no in-house IT) | Redundancy is basic but real: a second carrier or a wireless backup for internet, a documented manual fallback for CAD, and a backup radio console. Test the failover once a year and keep the result. |

Source Authority NENA-STA-040.2-2024: §5.3, §6.14, §6.15 NIST SP 800-53 Rev. 5: CP-2, CP-7, CP-8 Indiana statutes and administrative code: IC 36-8-16.7: Statewide 9-1-1 Services (redundancy)

## Approval Signatures

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Director | Anthony Reyes | /s/ Anthony Reyes | 2026-01-15 |
| Shared County IT | Priya Nair | /s/ Priya Nair | 2026-01-15 |

## Change History

| Version | Date | Author | Summary of Change |
| --- | --- | --- | --- |
| 1.0 | 2026-01-15 | Shared County IT | Initial issue |