Single-Point-of-Failure Assessment Record

Assessment date: 2026-02-13 Assessor (role): Director Scope: the Agency’s systems and paths reviewed for single points of failure, read against the System Redundancy Design Documentation Owner: Director Classification: Restricted

## 1. Executive Summary

This record identifies single points of failure in the Agency’s infrastructure, such as a lone cable route, a single power feed, or one core switch, and records the redundancy decision and priority for each. One high-impact single point of failure was found: the lone fiber path to the county network. A wireless failover circuit is planned. The single core switch is lower priority and is held with a cold spare.

## 2. Findings

| SPOF ID | Single point of failure (system / path) | Impact / priority | Redundancy treatment & decision |
| --- | --- | --- | --- |
| S-01 | Single fiber path to the county network (no diverse route) | High | Add an LTE/wireless failover circuit; manual cutover documented in the interim |
| S-02 | One core switch at the dispatch center | Med | Keep a cold-spare switch on site; accept the risk until a redundant pair is budgeted |

## 3. Remediation Tracking

| SPOF ID | Owner (role) | Action | Due date | Status |
| --- | --- | --- | --- | --- |
| S-01 | County IT Administrator | Order LTE failover router and document manual cutover | 2026-05-01 | In progress |
| S-02 | County IT Administrator | Purchase and stage a cold-spare core switch | 2026-06-15 | Open |

## Profile Scaling Notes

| PSAP Profile | Minimum-viable implementation |
| --- | --- |
| Small (1–5 dispatchers, no in-house IT) | The director walks the critical path once a year, notes anywhere a single failure would stop dispatch, and records the fix or the accepted risk. |

Source Authority NENA-STA-040.2-2024: §5.3 (Availability); §6.14 (Network Availability); §6.15 (Diversity) NIST SP 800-53 Rev. 5: CP-2, CP-7, PE-9, SC-5

## Sign-off

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Director | Tracy Holloway | /s/ Tracy Holloway | 2026-02-13 |
| County IT Administrator | Marcus Reed | /s/ Marcus Reed | 2026-02-13 |

## Revision History

| Version | Date | Author | Summary of Change |
| --- | --- | --- | --- |
| 1.0 | 2026-02-13 | Director | Initial issue |