Cellular / Mobile Device Security Standards

Authority: Adopted by [Agency Name] under the authority of [signing official / title]. Version: [#] Effective Date: [Date] Next Review: [Date] Owner: [Role title] «use a role, not a person’s name.» Classification: Restricted «Minimum for this standard: it prescribes private-APN and rogue-base-station controls, which are operational defensive detail. Stays Restricted. Your agency may raise but not lower this.»

How to use this template (delete this box before publishing) This template is ready to adopt. Replace every [BRACKETED] field with your agency’s information, follow the « » guidance notes, and delete every « » note and this box before you publish. Placeholder key: [BRACKETED] = fill in · [REQUIRED] (bold) = required field · « » = guidance to delete. Keep the body director-readable. Push technical detail and contact lists to an appendix. Source Authority, approval signatures, and revision history stay at the foot of the document.

## 1. Purpose

«What this standard prescribes and the policy it implements. A Standard states what must be true; it governs behavior.»

This standard sets the security requirements for cellular and mobile access to the Agency’s emergency-services applications. It states what a cellular connection must satisfy before it is allowed, covering device management, session handling, encryption of the cellular link, and detection of unauthorized base stations.

## 2. Scope

«What and who the standard binds; which systems or data it applies to.»

- Applies to: all cellular and mobile connections that reach Agency emergency-services applications, and the devices that make them.

- Implements: Wireless Network Policy; Mobile Device Management (MDM) Policy.

- Excludes: fixed wired connections, governed by the Access Control Policy; the deployed configuration values, recorded in the relevant configuration records.

## 3. Requirements

«The binding requirements, as numbered statements. Each must be testable. Use “The Agency shall…”.»

- Cellular access to emergency-services applications is formally approved before it is enabled, and is limited to mission-essential services.

- Cellular-connected devices are managed through mobile-device management or a private APN, so the Agency can enforce settings and revoke access.

- Cellular and broadband sessions carrying sensitive data are encrypted, using an agency VPN or application-level encryption.

- Non-essential cellular services are disabled when not in use, including after shift where a device is not on duty.

- Authentication for cellular access to emergency-services applications uses multifactor authentication or device certificates.

- Where the technology supports it, the Agency detects and prevents connections to unauthorized or rogue base stations, and monitors for interference.

- Cellular links are kept separate from real-time call-handling systems and are monitored for unusual activity.

- Encryption strength and approved algorithms follow the Encryption Policy; this standard does not restate cipher values.

## 4. Approved and Prohibited

«The concrete allow/deny lists this standard sets (e.g., approved algorithms, prohibited protocols). Keep the as-built evidence in the matching Configuration Record, not here.»

| Category | Approved | Prohibited |
| --- | --- | --- |
| Cellular transport | Private APN; agency VPN over LTE; application-level TLS | Public or consumer hotspot tethering to dispatch systems |
| Device management | MDM-enrolled, agency-managed devices | Unmanaged or personal devices for emergency-services applications |
| Session handling | Disable-when-idle; restrict to mission-essential services | Always-on non-essential cellular services on dispatch devices |
| Messaging of sensitive data | Approved secure channels | Consumer messaging apps for CJIS-regulated or sensitive data |
| Base-station security | Rogue-base-station detection where available | Connecting to unverified or unauthorized base stations |

## 5. Deviation Process

«Who may approve a deviation, how it is requested and recorded, and how long it lasts before review.»

A deviation (for example, a device or carrier that cannot meet a requirement) requires written approval from the [Director / Coordinator], a stated compensating control, and a review date. Deviations are recorded and expire after [period] unless renewed.

## 6. Review Schedule

«Review cadence and trigger events (annual at minimum; also on significant change). Name the owning role.»

This standard is reviewed at least annually and whenever the Agency’s cellular or mobile technology changes significantly, by the [Owner role].

## Profile Scaling Notes

One short paragraph per profile: what minimum-viable looks like for that reader and where to scale up. Keep all three if this ships as a template; delete the profiles that do not apply once adopted.

| PSAP Profile | Minimum-viable implementation |
| --- | --- |
| Small (1–5 dispatchers, no in-house IT) | Few cellular devices. Approve which devices may reach emergency-services applications, require MFA, and use the carrier’s or platform’s management to disable non-essential services. The director keeps the approval list. |
| Medium (6–25, shared county IT) | Shared county IT manages cellular devices through MDM or a private APN, encrypts sessions with a VPN, and restricts devices to mission-essential services. |
| Large (25+, dedicated IT/security) | A private APN and MDM enforce device management, session encryption, and disable-when-idle across the fleet, with rogue-base-station detection and monitoring where the technology supports it. |

Source Authority List only the authorities that map to content in this artifact. Group by authority; order sections and control IDs numerically. No padding. Replace the lines below with the citations that actually apply. NENA-STA-040.2-2024: §6.3.4 (Broadband Cellular); §6.7.1 (Remote Access Device Security); §6.22 (Encryption) NIST SP 800-53 Rev. 5: AC-18, AC-19, SC-8, SC-40

## Approval Signatures

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| [Approving official / title] |  |  |  |
| [Owner role] |  |  |  |

## Revision History

| Version | Date | Author | Summary of Change |
| --- | --- | --- | --- |
| [#] | [Date] | [Role] | [Initial issue] |

Drafter’s note (internal, delete before publishing): The Agency fills [Agency Name], version, dates, and the Owner role. Confirm CJIS applicability: keep the / CJIS marker and the FIPS-validated-module requirement only if the Agency stores or transmits CJIS-regulated data on systems it controls. Key generation, custody, and rotation are governed by the Key & Certificate Management Policy, not restated here. As-built proof of conformance is recorded in the Encryption Implementation Verification Records. Cellular / Mobile Device Security Standards for category 4.E. Restricted (raised from Internal): the standard prescribes private-APN and rogue-base-station controls, which are operational defensive detail, not public best practice. It implements the Wireless Network Policy and the Mobile Device Management (MDM) Policy; encryption strength lives in the Encryption Policy. Confirm the Agency’s cellular management method (MDM or private APN), the encryption of cellular sessions, and whether rogue-base-station detection is available.