Firewall Ruleset Documentation

Authority: Adopted by Tri-River Regional 911 Center under the authority of Dr. Lisa Park, Director. Version: 1 Effective Date: 2026-01-26 Next Review: 2027-01-26 Owner: Information Security Analyst Classification: Restricted

## 1. Purpose and Scope

This record documents the as-built firewall, session border controller, and edge-appliance rules that enforce the agency's network boundary. It implements the Network Segmentation Design Document and follows a default-deny posture: traffic is denied unless a rule explicitly allows it. It is change-controlled and covers every perimeter device listed below.

## 2. System Identification

| Device / appliance | Asset ID | Role | Zones bordered |
| --- | --- | --- | --- |
| Next-gen firewall cluster (Center A) | FW-A1/A2 | Boundary control, default-deny, IPS | External / DMZ / internal |
| Next-gen firewall cluster (Center B) | FW-B1/B2 | Boundary control at second center | External / DMZ / internal |
| Session border controllers (pair) | SBC-01/02 | SIP boundary to dual ESInet | Voice / external |
| Edge routers | RTR-01/02 | Upstream carrier handoff | External / perimeter |

## 3. Current Configuration

| Zone / interface | Default posture | Allowed flow (source → destination) | Port / protocol | Justification |
| --- | --- | --- | --- | --- |
| External interface | Deny all | Carrier A/B ESInet → SBC pair | 5060–5061 / SIP | NG911 delivery across two centers |
| DMZ | Deny all | DMZ services → internal via proxy | 443 / HTTPS | Partner and public services |
| Internal | Deny all | Dispatch → GIS / RMS / ALI | 443 / HTTPS | Integrated call handling |
| Internal | Deny all | Admin → critical VLANs | denied | Least privilege; no admin-to-critical |

## 4. Approved Rules and Exceptions

| Rule / setting | Approved baseline | Exception (if any) | Approved by (role) | Date |
| --- | --- | --- | --- | --- |
| Default-deny inbound | Enabled on all interfaces and both SBC pairs | None | Information Security Analyst | 2026-01-26 |
| Peer-to-peer between endpoints | Blocked | Approved secure file-transfer service, logged | Information Security Analyst | 2026-01-26 |
| Inter-center replication | Allowed on dedicated encrypted link | Documented, reviewed quarterly | Information Security Analyst | 2026-01-26 |

## 5. Validation and Review

The ruleset is reviewed at least quarterly by the Information Security Analyst: each rule is checked against the Network Segmentation Design Document, obsolete or unused rules are removed, and the default-deny posture is confirmed. Perimeter devices log allowed and denied connections and alert on unauthorized attempts. Each rule change is recorded in the Change History and cross-referenced to the Change Request & Approval Log.

## Profile Scaling Notes

| PSAP Profile | Minimum-viable implementation |
| --- | --- |
| Large (25+, dedicated IT/security) | Redundant next-generation firewalls and SBCs across both centers, allow-lists managed under change control, logging and alerting fed to monitoring, and scheduled rule recertification. |

Source Authority NENA-STA-040.2-2024: §6.3.5, §6.10, §6.10.1, §6.10.2, §6.11 NIST SP 800-53 Rev. 5: AC-4, AU-12, CM-6, CM-7, SC-7

## Approval Signatures

| Role | Name | Signature | Date |
| --- | --- | --- | --- |
| Director | Dr. Lisa Park | /s/ Dr. Lisa Park | 2026-01-26 |
| Information Security Analyst | Raymond Osei | /s/ Raymond Osei | 2026-01-26 |

## Change History

| Version | Date | Author | Summary of Change |
| --- | --- | --- | --- |
| 1 | 2026-01-26 | Raymond Osei | Initial adoption |